Lab Write-Up Packet Tracer — Configure DHCP on a Wireless Router

| Author | Watipaso Jenda | Platform | Cisco Packet Tracer |
| --- | --- | --- | --- |
| Course | Networking Basics — Cisco NetAcad | Difficulty | Beginner |
| Lab File | Configure_DHCP_on_a_Wireless_Router.pka | Status | ✔ Completed |

# 1. Lab Overview

This lab builds directly on the previous wireless router configuration lab by going deeper into DHCP scope management. Rather than using default router settings, the goal here is to understand what default DHCP settings look like, then deliberately change the router's IP address and DHCP pool to a custom network range, and observe how that change affects connected clients in real time.

Objectives covered:

- Connect 3 PCs to a wireless router using straight-through Ethernet cables

- Observe and record the default DHCP settings and IP addressing

- Change the router's IP address from 192.168.0.1 to 192.168.5.1

- Update the DHCP starting address to 192.168.5.126 and max users to 75

- Enable DHCP on all three client PCs and verify address assignment

- Verify full connectivity via ping between clients and router

# 2. Network Topology

A simple flat network — three PCs connected directly to a wireless router via Ethernet, no additional switches or ISP equipment in this lab.

- Wireless Router (192.168.5.1)

- │         │         │

- FastEth0  FastEth1  FastEth2

- │         │         │

- PC0       PC1       PC2

- (.126)     (.127)    (.128)

# 3. IP Addressing Table

Note: The table shows both the initial default addresses and the final addresses after the DHCP scope change. This reflects the two-phase nature of the lab.

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Wireless Router (initial) | 192.168.0.1 | 255.255.255.0 | ISP Assigned |
| Wireless Router (updated) | 192.168.5.1 | 255.255.255.0 | ISP Assigned |
| PC0 (before change) | 192.168.0.100 | 255.255.255.0 | 192.168.0.1 |
| PC0 (after change) | 192.168.5.126 | 255.255.255.0 | 192.168.5.1 |
| PC1 | 192.168.5.127 | 255.255.255.0 | 192.168.5.1 |
| PC2 | 192.168.5.128 | 255.255.255.0 | 192.168.5.1 |

# 4. Configuration Steps

## Part 1 — Physical Setup

- Added three generic PCs to the topology

- Connected PC0, PC1, and PC2 to the wireless router using straight-through cables

- Waited for amber link lights to turn green before proceeding

## Part 2 — Observe Default DHCP Settings

Step 1: Set PC0 to DHCP in IP Configuration. It automatically received 192.168.0.100 — confirming the default router range starts at .100.

Step 2: Opened a web browser on PC0 and navigated to 192.168.0.1 (the default gateway). Logged in with default credentials (admin / admin).

Step 3: Scrolled through the Basic Setup page to observe default settings — DHCP enabled, starting address 192.168.0.100, default pool size. Recorded these before making any changes.

## Part 3 — Change Router IP Address

Step 4: Under Router IP Settings, changed the IP address from 192.168.0.1 to 192.168.5.1. Clicked Save Settings.

Observed: The browser immediately showed an error page. This is expected — the router had moved to a new network (192.168.5.1) and PC0's old IP (192.168.0.100) was no longer on the same network. The browser had no valid path to reach the router's new address.

Step 5: Closed the browser. In IP Configuration, switched PC0 to Static, then back to DHCP to force a fresh DHCP request. PC0 received a new address on the 192.168.5.1 network. Navigated to 192.168.5.1 — router GUI accessible again.

## Part 4 — Update DHCP Scope

Step 6: In the router GUI, the DHCP start address had automatically updated to 192.168.5.100 (matching the new router network). Changed it to 192.168.5.126. Changed Maximum Number of Users to 75. Saved settings.

Step 7: Renewed PC0's IP again (Static → DHCP). PC0 received 192.168.5.126 — the first address in the new pool. Confirmed via ipconfig in Command Prompt.

## Part 5 — Enable DHCP on PC1 and PC2

- Clicked PC1 → Desktop → IP Configuration → selected DHCP

- PC1 received 192.168.5.127 automatically

- Repeated for PC2 — received 192.168.5.128

# 5. Key Learning & Observations

## What Happens When You Change a Router's IP Mid-Session

The most important moment in this lab was what happened after saving the new router IP (192.168.5.1). The browser returned an error immediately — not a credentials error, not a timeout, but a connection failure. The reason is straightforward once you understand routing: PC0's IP was 192.168.0.100, and the router had just moved to 192.168.5.1. Those are two different networks. PC0 had no route to reach 192.168.5.1 because its own address was still on the 192.168.0.1 network.

The fix — toggling PC0 from Static back to DHCP — forced a new DHCP Discover broadcast. The router (now on 192.168.5.1) responded with an address on the new network, and the connection was restored. This is a real-world scenario: any time a router or gateway IP is changed in a live network, connected clients may lose access until they renew their leases.

## DHCP Scope — What the Numbers Mean

Setting the DHCP start address to 192.168.5.126 with a maximum of 75 users means the router will assign addresses from 192.168.5.126 to 192.168.5.200. The addresses from 192.168.5.1 to 192.168.5.125 are reserved — available for static assignment to servers, printers, or other infrastructure devices that should not change IP addresses. This is standard practice in both home and enterprise environments: DHCP for dynamic clients, static or reserved addresses for fixed infrastructure.

# 6. Verification

| Test | Result | Notes |
| --- | --- | --- |
| PC0 receives default DHCP address (192.168.0.100) | ✔ Pass | Default router range confirmed |
| Router IP changed to 192.168.5.1 | ✔ Pass | Browser error on save — expected behaviour |
| PC0 IP renewed after router IP change | ✔ Pass | Static → DHCP toggle to force renewal |
| DHCP start address changed to 192.168.5.126 | ✔ Pass | Max users set to 75 |
| PC0 receives 192.168.5.126 | ✔ Pass | First address in new pool |
| PC1 receives 192.168.5.127 | ✔ Pass | DHCP enabled manually |
| PC2 receives 192.168.5.128 | ✔ Pass | DHCP enabled manually |
| PC2 pings router (192.168.5.1) | ✔ Pass | Connectivity verified |
| PC2 pings PC1 (192.168.5.127) | ✔ Pass | Cross-client connectivity confirmed |

# 7. Reflection

This lab reinforced a concept that directly extends the previous lab: DHCP pools are not fixed. The network range, starting address, and pool size are all administrator-controlled decisions that affect every device on the network. Changing the router IP mid-session and watching clients lose connectivity — then recovering by forcing a DHCP renewal — made the mechanics of DHCP concrete in a way that reading about it doesn't.

The split between the DHCP pool (192.168.5.126–.200) and the reserved lower range (192.168.5.1–.125) is a design pattern worth remembering. In any real network you would reserve addresses for infrastructure, assign DHCP for endpoints, and document which devices hold which static addresses. That habit starts at the small scale and scales to enterprise environments directly.

# 8. Lab Files

- Configure_DHCP_on_a_Wireless_Router.pka — Packet Tracer file with all configurations applied

- Configure_DHCP_on_a_Wireless_Router.html — Original Cisco NetAcad instruction manual

- Lab02_DHCP_Wireless_Router_Writeup.docx — This write-up