Lab Write-Up Packet Tracer — Identify MAC and IP Addresses

| Author | WATIPASO JENDA | Platform | Cisco Packet Tracer |
| --- | --- | --- | --- |
| Course | Networking Basics — Cisco NetAcad | Difficulty | Beginner |
| Lab File | Identify_MAC_and_IP_Addresses.pka | Status | ✔ Completed |

# 1. Lab Overview

This lab uses Packet Tracer Simulation mode to observe how MAC and IP addresses behave as a PDU travels across a network. No device configuration is required — the focus is entirely on reading and interpreting packet headers at each hop, for both local and remote network communication.

Objectives:

- Observe PDU header information during local network communication

- Observe PDU header information during remote network communication

- Understand why MAC addresses change at each hop while IP addresses remain constant

# 2. Network Topology

Two networks connected by a router — a wired local network (172.16.31.x) and a remote wireless network (10.10.10.x).

Local:  172.16.31.3 → Switch 2 → 172.16.31.2

Remote: 172.16.31.3 → Switch 2 → Router → Switch 1 → Access Point → 10.10.10.2

# 3. Part 1 — Local Network Communication

Pinged 172.16.31.2 from 172.16.31.3. Both on the same network — no router required. Stepped through each hop in Simulation mode recording PDU headers.

| At Device | Src MAC | Dst MAC | Src IPv4 | Dst IPv4 |
| --- | --- | --- | --- | --- |
| 172.16.31.3 | 0060.7036.2849 | 000C.85CC.1DA7 | 172.16.31.3 | 172.16.31.2 |
| Switch 2 | 0060.7036.2849 | 000C.85CC.1DA7 | N/A | N/A |
| 172.16.31.2 (in) | 000C.85CC.1DA7 | 000C.85CC.1DA7 | 172.16.31.3 | 172.16.31.2 |
| 172.16.31.2 (out) | 0060.7036.2849 | 0060.7036.2849 | 172.16.31.2 | 172.16.31.3 |

Key observation: MAC addresses stayed consistent — no router involved. The switch forwarded using MACs only and has no IP awareness.

# 4. Part 2 — Remote Network Communication

Pinged 10.10.10.2 from 172.16.31.3. Destination is on a different network — packet must pass through the router. First hop destination MAC is the router's interface, not the final device.

| At Device | Src MAC | Dst MAC | Src IPv4 | Dst IPv4 |
| --- | --- | --- | --- | --- |
| 172.16.31.3 | 00D0.D311.C788 | 00D0.BA8E.741A | 172.16.31.3 | 10.10.10.2 |
| Switch 2 | 0060.7036.2849 | 00D0.BA8E.741A | N/A | N/A |
| Router (in) | 0060.7036.2849 | 00D0.BA8E.741A | 172.16.31.3 | 10.10.10.2 |
| Router (out) | 00D0.588C.2401 | 0060.2F84.4AB6 | 172.16.31.3 | 10.10.10.2 |
| Switch 1 | 00D0.588C.2401 | 0060.2F84.4AB6 | N/A | N/A |
| Access Point | N/A | N/A | N/A | N/A |
| 10.10.10.2 | 0060.2F84.4AB6 | 00D0.588C.2401 | 10.10.10.2 | 172.16.31.3 |

Key observation: IP addresses never changed across any hop. MAC addresses were completely rewritten at the router. This is Layer 2 vs Layer 3 behaviour — IPs are end-to-end, MACs are hop-to-hop.

# 5. Key Observations

| Observation | What It Means |
| --- | --- |
| IP addresses never changed | IPs are end-to-end — same source and destination the entire journey |
| MAC addresses changed at the router | MACs are hop-to-hop — rewritten at every Layer 3 device |
| Switch shows N/A for IP | Switches operate at Layer 2 only — they read MACs, not IPs |
| Dst MAC on first hop = router interface MAC | PC sends to router's MAC because destination is on a remote network |
| Local comm: no gateway needed | Same-network devices communicate directly using MAC addresses only |

# 6. Key Learning

IP = end-to-end. Never changes. Identifies original source and final destination for the entire journey.

MAC = hop-to-hop. Only meaningful on the local segment. Rewritten at every router boundary with new source and destination MACs for the next leg.

Switches are Layer 2 only. Forward frames by MAC address — completely unaware of IP addressing.

Remote destination → first hop MAC = router. The sending device recognises the destination is not local and addresses the frame to the router's MAC to be forwarded onward.

# 7. Lab Files

- Identify_MAC_and_IP_Addresses.pka — Completed Packet Tracer file

- Identify_MAC_and_IP_Addresses.html — Original Cisco NetAcad instruction manual

- Lab04_Identify_MAC_and_IP_Addresses_Writeup.docx — This write-up