Lab Write-Up Packet Tracer — Examine NAT on a Wireless Router

| Author | Watipaso Jenda | Platform | Cisco Packet Tracer |
| --- | --- | --- | --- |
| Course | Networking Basics — Cisco NetAcad | Difficulty | Beginner |
| Lab File | Examine_NAT_on_a_Wireless_Router.pka | Status | ✔ Completed |

# 1. Lab Overview

This lab examines how Network Address Translation (NAT) works on a home wireless router. Unlike the previous two labs which focused on configuration, this lab is primarily observational — the goal is to understand the boundary between private and public IP addressing, and to watch NAT translate packet headers in real time using Packet Tracer's Simulation mode.

NAT is one of the most fundamental concepts in modern networking. It is the mechanism that allows millions of home and office networks to share a small pool of public IP addresses — without it, the internet as it exists today would not be possible.

Objectives covered:

- Connect 4 PCs to a wireless router and configure all clients for DHCP

- Examine the router's WAN (public) IP assigned by the ISP

- Examine the router's LAN (private) IP and DHCP pool

- Use Simulation mode to generate HTTP traffic from a PC to a web server

- Observe NAT translation by examining inbound and outbound packet headers

- Confirm that the source IP changes as packets cross the router boundary

# 2. Network Topology

The topology has two distinct zones separated by the wireless router — a private internal network and a simulated public internet. NAT operates at the boundary between these two zones.

- [ INTERNET / PUBLIC ZONE ]

- |

- ciscolearn.nat.com  (web server — public IP)

- |

- [ ISP DHCP Server ]

- |

- Wireless Router WAN  (public IP — ISP assigned)

- ═══════ NAT BOUNDARY ═══════

- Wireless Router LAN  (192.168.0.1 — private)

- |       |       |       |

- PC0     PC1     PC2     PC3

- (private IPs via DHCP — 192.168.0.x)

# 3. IP Addressing Table

Note: Private IPs for the 4 PCs are shown as 192.168.0.x as the exact DHCP-assigned addresses were not recorded during the lab. The router's public WAN IP was ISP-assigned and visible under Status > Router in the router GUI.

| Device | IP Address | Address Type | Notes |
| --- | --- | --- | --- |
| Wireless Router (WAN) | ISP Assigned | Public | Assigned by ISP DHCP — visible on internet |
| Wireless Router (LAN) | 192.168.0.1 | Private | Gateway for all internal clients |
| PC0 | 192.168.0.x | Private | DHCP assigned — cannot cross internet |
| PC1 | 192.168.0.x | Private | DHCP assigned — cannot cross internet |
| PC2 | 192.168.0.x | Private | DHCP assigned — cannot cross internet |
| PC3 | 192.168.0.x | Private | DHCP assigned — cannot cross internet |
| ciscolearn.nat.com | Public (server) | Public | Destination web server on simulated internet |

# 4. What is NAT?

NAT — Network Address Translation — is a process performed by routers that translates private IP addresses to a public IP address (and back again) as packets cross the boundary between a private network and the public internet.

Private IP addresses (ranges like 192.168.x.x, 10.x.x.x, 172.16.x.x–172.31.x.x) are not routable on the public internet. Every home router, office router, and mobile hotspot uses NAT to bridge this gap — devices inside the network use private addresses to communicate with each other, but when traffic leaves to the internet, the router replaces the private source IP with its own public IP. When the response comes back, the router translates it back and delivers it to the correct internal device.

Why this matters for networking and security: NAT effectively hides internal device addresses from the public internet, which provides a basic layer of obscurity. However, NAT is not a security mechanism — it is an address conservation mechanism. Understanding this distinction is important for anyone working in network security.

# 5. Configuration Steps

## Part 1 — Examine Router WAN Configuration

- Added PC0 and connected it to the wireless router via straight-through cable

- Set PC0 to DHCP — received a private IP address from the router

- Noted the default gateway IP, opened browser, navigated to router GUI

- Clicked Status > Router — scrolled to Internet Connection

- Confirmed the WAN IP was a public address assigned by the ISP DHCP server

## Part 2 — Examine Router LAN Configuration

- Clicked Status > Local Network in the router GUI

- Confirmed the LAN IP (192.168.0.1) — a private address

- Examined DHCP server settings — confirmed the private address range assigned to clients

- Noted that these private addresses cannot be routed across the internet without NAT

## Part 3 — Connect Remaining PCs

- Added PC1, PC2, and PC3 — connected each via straight-through cable to the router

- Set each PC to DHCP — all received private IP addresses from the router's DHCP pool

- Ran ipconfig /all in Command Prompt on each PC to verify IP configuration

## Part 4 — View NAT Translation in Simulation Mode

- Switched to Simulation mode (stopwatch tab, lower right)

- Clicked Show All/None to clear all visible events

- Opened Edit Filters > Misc tab — checked TCP and HTTP only

- Added a Complex PDU: source = PC, destination = ciscolearn.nat.com, application = HTTP, source port = 1000, interval = 120s periodic

- Moved simulation panel to see full topology, clicked Play to observe traffic flow

## Part 5 — Examine Packet Headers

- Double-clicked event lines in the Simulation Panel to open packet details

- Clicked envelope in the work area to view PDU information

- Inbound PDU tab: SRC IP showed the PC's private address (192.168.0.x)

- Outbound PDU tab: SRC IP showed the router's public WAN IP — NAT had translated it

- Clicked through multiple event lines to observe the full round-trip translation

# 6. NAT Translation — What the Headers Showed

The following table summarises the packet header changes observed during the simulation — this is the core of what NAT does at each stage of the journey:

| Direction | Source IP (SRC) | Destination IP | What this means |
| --- | --- | --- | --- |
| PC → Router (inbound) | 192.168.0.x (private) | ciscolearn.nat.com (public) | Private IP leaving the PC |
| Router → Internet (outbound) | ISP Public IP | ciscolearn.nat.com (public) | NAT replaces private with public IP — packet can now cross internet |
| Internet → Router (return) | ciscolearn.nat.com (public) | ISP Public IP | Response arrives at router's public IP |
| Router → PC (inbound) | ciscolearn.nat.com (public) | 192.168.0.x (private) | Router translates back to private IP and delivers to correct PC |

# 7. Verification

| Test | Result | Notes |
| --- | --- | --- |
| Router WAN shows ISP-assigned public IP | ✔ Pass | Confirmed under Status > Router > Internet Connection |
| Router LAN shows private IP (192.168.0.1) | ✔ Pass | Confirmed under Status > Local Network |
| All 4 PCs receive private DHCP addresses | ✔ Pass | Verified via IP Configuration on each PC |
| Simulation mode configured for TCP/HTTP only | ✔ Pass | Edit Filters → Misc tab → TCP and HTTP checked |
| Complex PDU created from PC to ciscolearn.nat.com | ✔ Pass | HTTP, source port 1000, periodic 120s |
| Inbound SRC IP shows private address | ✔ Pass | Private IP visible before NAT translation |
| Outbound SRC IP shows public address | ✔ Pass | NAT replaced private IP with router's public IP |
| SRC IP change observed across router boundary | ✔ Pass | Core NAT behaviour confirmed in packet headers |

# 8. Key Learning & Observations

## Private vs Public Addressing — The Boundary is Real

Before this lab, the concept of private and public IP addresses was abstract. Watching it happen in Simulation mode made it concrete: the same packet has a different source IP depending on which side of the router you're looking at. On the inside, it's the PC's private address. On the outside, it's the router's public address. The router is performing this substitution silently on every single packet that crosses the boundary.

This is why devices on your home network can reach the internet even though their 192.168.x.x addresses have no meaning outside your house. The router is the only device with a real public IP — everyone behind it shares it.

## NAT is Not Security

A common misconception is that NAT provides network security because internal devices are hidden behind a public IP. This is only partially true. NAT does obscure internal addresses, but it was designed for address conservation, not protection. A determined attacker does not need to know your internal IPs to attack your network — they target the public IP and the open ports on it.

Real security comes from firewalls, access control lists, port filtering, and proper network segmentation — not NAT alone. Understanding this distinction early is important for anyone heading into cybersecurity.

## Simulation Mode as a Diagnostic Tool

Packet Tracer's Simulation mode is a simplified version of what real packet analysis tools like Wireshark do. The ability to pause traffic, click into individual packets, and read header fields is a foundational skill for network troubleshooting and security analysis. Every firewall rule, every IDS alert, and every network forensic investigation comes down to reading and interpreting packet headers. This lab was the first time that process was used hands-on.

# 9. Lab Files

- Examine_NAT_on_a_Wireless_Router_Instructions.pka — Completed Packet Tracer file

- Examine_NAT_on_a_Wireless_Router_Instructions.html — Original Cisco NetAcad instruction manual

- Lab03_NAT_Wireless_Router_Writeup.docx — This write-up