Lab Write-Up Packet Tracer — Observe Traffic Flow in a Routed Network

| Author | WATIPASO JENDA | Platform | Cisco Packet Tracer |
| --- | --- | --- | --- |
| Course | Networking Basics — Cisco NetAcad | Difficulty | Beginner |
| Lab File | Observe_Traffic_Flow_in_a_Routed_Network.pka | Status | ✔ Completed |

# 1. Lab Overview

This lab demonstrates the real-world business case for routing — why a flat single-network design becomes a problem at scale and how separating departments into routed subnets improves performance and reduces unnecessary broadcast traffic. The lab has three parts: observe an unrouted network, reconfigure it to use routing, then observe the improvement.

Objectives:

- Observe how ARP broadcasts flood an entire unrouted LAN

- Reconfigure network cabling to connect each department switch directly to a router interface

- Renew IP addresses on hosts to receive new subnet assignments from the Edge router

- Observe how routing contains ARP broadcasts to relevant subnets only

- Understand the performance and security benefits of network segmentation

# 2. Topology

The network represents XYZ LLC — a growing company with three departments: Accounting, Finance, and Sales. An Edge router connects the LAN to the internet (ISP cloud).

Before routing:  All three department switches chained together on a single flat network (192.168.1.0/24)

After routing:   Each switch connected directly to a dedicated router interface — three separate subnets

# 3. Network Comparison

|  | Unrouted (Before) | Routed (After) |
| --- | --- | --- |
| Network structure | Single flat network — all departments on 192.168.1.0/24 | Three separate networks per department |
| ARP broadcast reach | Every device on the entire network | Only devices on the same subnet |
| Accounting network | 192.168.1.0/24 (shared) | 192.168.1.0/24 |
| Finance network | 192.168.1.0/24 (shared) | 192.168.2.0/24 |
| Sales network | 192.168.1.0/24 (shared) | 192.168.3.0/24 |
| Switch connections | Switches chained to each other | Each switch connected directly to router interface |
| IP assignment | Static / existing DHCP | DHCP from Edge router interfaces |

# 4. Configuration Steps

## Part 1 — Observe Unrouted Traffic

- Cleared ARP cache on Sales 1 using arp -d in Command Prompt

- Switched to Simulation mode, pinged Sales 1 from Sales 2

- Stepped through PDUs using Capture then Forward

- Observed: first PDU was an ARP broadcast (dst MAC = FFFF.FFFF.FFFF) — processed by every host and the router interface on the entire network

- After ARP resolved, ICMP echo-request was sent and ping succeeded

## Part 2 — Reconfigure Network to Route Between LANs

- Clicked the green triangle on the Accounting-to-Finance cable at the Accounting switch end

- Dragged and reconnected that cable end to Edge router GigabitEthernet 1/0

- Repeated for the Finance-to-Sales cable — connected to next available GigabitEthernet port

- Opened Command Prompt on all Finance and Sales hosts, ran ipconfig /renew

- Finance hosts received new addresses on 192.168.2.0/24

- Sales hosts received new addresses on 192.168.3.0/24

- Accounting hosts remained on 192.168.1.0/24 — no change needed

## Part 3 — Observe Routed Traffic

- Cleared ARP cache on Sales 2, switched to Simulation mode

- Pinged Sales 1 from Sales 2 and stepped through PDUs

- Observed: ARP broadcast now only processed by Sales 1 and the router interface on the Sales subnet

- Accounting and Finance devices received no broadcast traffic at all

- Pinged hosts across departments and internet — router forwarded traffic between subnets correctly

# 5. Verification

| Test | Result | Notes |
| --- | --- | --- |
| ARP broadcast hit all devices in unrouted network | ✔ Pass | All hosts and router interface processed ARP request |
| Sales 2 ping to Sales 1 succeeded (unrouted) | ✔ Pass | ICMP echo-request confirmed after ARP resolution |
| Accounting switch recabled to router GigEth 1/0 | ✔ Pass | Cable moved from switch-to-switch to router interface |
| Finance switch recabled to router GigEth interface | ✔ Pass | Separate router interface per department |
| ipconfig /renew run on Finance and Sales hosts | ✔ Pass | New IPs received from Edge router DHCP |
| Finance hosts received 192.168.2.0/24 addresses | ✔ Pass | Confirmed via ipconfig after renewal |
| Sales hosts received 192.168.3.0/24 addresses | ✔ Pass | Confirmed via ipconfig after renewal |
| ARP broadcast contained to Sales subnet only | ✔ Pass | Only Sales 1 and router interface processed ARP |
| Ping between departments and internet succeeded | ✔ Pass | Router forwarded traffic between subnets correctly |

# 6. Key Learning

Broadcast domains and why they matter. In a flat unrouted network, every ARP request is processed by every device. With 150 devices this happens constantly — eating into bandwidth and processing capacity that should be handling real work. Routing creates boundaries that contain broadcasts to relevant subnets only.

Routing improves performance. After segmentation, a Sales ARP request only touched Sales devices and the router — not Accounting or Finance. At enterprise scale this difference is significant.

Routing improves security. Separate subnets mean departments are isolated by default. Traffic between them must pass through the router, where access control rules can be applied. A flat network has no natural boundary to enforce policies between departments.

Routing is appropriate for growth. A flat single-network design is acceptable for very small networks. Once a network grows — more devices, more departments, more traffic — routing becomes necessary, not optional.

ipconfig /renew forces DHCP renewal. When the physical network changed, hosts needed new IP addresses matching their new subnet. Rather than waiting for leases to expire, ipconfig /renew immediately requested a new address from the router's DHCP service — a practical troubleshooting command worth knowing.

# 7. Lab Files

- Observe_Traffic_Flow_in_a_Routed_Network.pka — Completed Packet Tracer file

- Lab05_Observe_Traffic_Flow_Routed_Network_Writeup.docx — This write-up